ANSWERS

Lab 7

Elevation Surface Visualization,

Analysis, and Modeling

Deliverables:

  - Question 1-9

  - Graphics 1-6

  - Submit as one PDF on Canvas.

QUESTION 1: What are slope, aspect, and hillshade? (Hint: Read the Help if you need. Cite where necessary). What are these tools doing?

Courtesy of ESRI:

Slope is the gradient or steepness over cells.

The aspect identifies the compass direction that the downhill slope faces for each location.

The HillShade tool creates a shaded relief raster from a raster. The illumination source is considered to be at infinity.

The contour tool creates a feature class of contours from a raster surface. Contours are polygons of constant elevation.

QUESTION 2: What are the settings that determine where the shadows fall on the hillshade?

Azimuth – the azimuth angle of the light source. The azimuth is expressed in positive degrees from 0 to 360, measured clockwise from north.

Altitude – the angle of the light source above the horizon. The altitude is expressed in positive degrees, with 0 degrees at the horizon and 90 degrees directly overhead. The default is 45 degrees.

Model Shadows - Type of shaded relief to be generated.

Z Factor - Number of ground x,y units in one surface z-unit. The z-factor adjusts the units of measure for the z-units when they are different from the x,y units of the input surface. The z-values of the input surface are multiplied by the z-factor when calculating the final output surface.

QUESTION 3: What fields are in the TwoViewpoints layer?

ObjectID and Shape

QUESTION 4: Which point has a larger viewshed? What is the elevation of this point?

Point 1 has a larger viewshed, at an elevation of 8084 ft.

QUESTION 5: What tools are run in the model?

Raster Calculators are run on Slope, McDEM_10m, as well as goodslope, goodaspect, and goodelev afterwards. Reclassify is run on Aspect.

QUESTION 6: What are the parameters and/or equations used in these tools? Write up what these parameters and equations are doing in plain English (short paragraph).

Raster Calculator (3) is selecting raster cells where slope is less than 40.

The reclassify tool for aspect reclassifies values between -1 and 0 to 0, 0 and 22.5 to 1, 22.5 and 337.5 to 0, 337.5 to 359.9999 to 1.

Raster Calculator is selecting cells where the elevation (McDEM_10m) is greater than 7875.

The final raster calculator multiplies the values of goodelev by goodslope by goodaspect.

QUESTION 7: Based on the viewsheds and denning habitat, suggest which viewshed would be best for observing denning habitat and why?

According to the raster calculations in the model listed above, viewshed 1 is best for observing denning habitat. Its output raster has a count of 773852, compared to the count of 23944 for viewshed 2.

QUESTION 8: Justify your symbolization method and color choice, why did you use Classify or Stretch? If you chose Classify, why switch to Defined Interval? How would a legend look different because of your choices?

I chose the stretch method of symbology. Stretching is continuous, and I think this makes more sense for elevation than classifying it into discrete bands. With a discrete scheme, we would have more accuracy in the visualization with the more bands we add, as the amount of values a single band has to represent is decreased. However, as we add more bands, we would start to approximate a continuous scheme.

I chose a color scheme that maps areas of low elevation to green, and areas of high elevation to brown. I chose this because there is typically less vegetation on top of hills where elevation is highest.

QUESTION 9: What is “draping” a layer in ArcScene? What is the layer being draped onto?

According to ESRI, draped layers “use other layers as elevation sources. Drape a layer to show it on a 3D surface. For example, you might drape an aerial photo and its associated features on a mountaintop.” The layer is being draped onto the DEM.

GRAPHIC 1: Take a screenshot of your new attribute table for the TwoViewpoints layer.

GRAPHIC 2: Take a screenshot of the WolverineElevations layer. You may want to turn off the other layers if they are obscuring your results.

GRAPHIC 3: Include a screenshot of this map in your deliverables

GRAPHIC 4: Put this map in Layout view. Add a title and scale bar. No need to add a legend because legends in ArcGIS don’t take into account the transparency you added, so your colors won’t match your legend anyways. Export the map as a .pdf.

GRAPHIC 5: Screenshot of 3D Hillshade

GRAPHIC 6: Screenshot of 3D Ortho Photo with trail and viewpoints on top